To set the Signal Hawk model in the WinCE registry, follow these steps:

1. Copy the appropriate registry file to \FlashFX Disk via ActiveSync.

The files are named for the model (SH36S.reg, SH362S.reg, etc).

2. Update the persistent registry on the unit. This will require either a usb

mouse or the MyMobile remote desktop utility from a remote PC. In either case

follow the remaingin steps.

2.1 Exit the Signal Hawk application to the WinCE desktop.

2.2 Navigate to the control panels (figures 1, 2, & 3).

2.3 Run the "Configuration" control panel.

2.4 Make sure ADSLOAD.REG is selected, then from the Action menu, select

import, and navigate to the \FlashFX Disk (figure 4).

2.5 Select the appropriate registry file (figure 5) and click OK.

2.6 Close the Configuration control panel.

2.7 Run the "ADS Persistent Registry" control panel, and click the "Save

Current Registry" button.

2.8 Click "Yes" to verify that you want to overwrite the persistent registry

file (figures 6 & 7).

2.9 Close the control panel & all the windows till you get back to the desktop.

2.10 Cycle the power on the Signal Hawk to activate the new persistent registry.

After this process, the SignalHawk.exe application (version 1.1.20080529(beta) &

later) should show the model as specified, and restrict the UI to valid modes of

operation. For example, a model SH-36S will show only the Spec and and Power

Measure modes & measurements.

Screens:

Figure 1

Figure 2

Figure 3

Figure 4

Figure 5

Figure 6

Figure 7